K-WASH

Video - Developing a climate analysis for WASH projects in Kenya

# Opening

Climate change is currently one of the greatest threats to global development and the equilibrium of ecosystems.

Developing countries, which are often the least responsible for emissions, are nevertheless suffering the most severe effects, due to a lack of resources for adaptation.

In Kenya, its impacts are increasingly being felt droughts are becoming longer, rainfall more irregular and unpredictable, floods more frequent, while temperatures continue to rise. These upheavals are undermining communities, natural resources and the very foundations of sustainable development.

Visual1 : Kenya's climate map

From near-equatorial to alpine, Kenya is a climatic mosaic.

The IPCC's climate projections point to future upheavals in distribution, with a generally warmer climate, but often wetter and more irregular, with concentrated rainfall, more intense floods, and droughts still possible.

Water will thus become an even more central issue, and it is essential to take past and future climate changes into account in the design and maintenance of water supply, sanitation and hygiene systems.

Visual2 : Visual IPCC

At the same time, the numerous research projects carried out by the IPCC, in particular as part of the preparation of the 6thAR6 report, have made it possible :

On the one hand, to aggregate and consolidate past climate data since 1940,

On the other hand, to develop global climate models for different climate evolution scenarios, the simulation results of which for the 2100 horizon are freely available on the Internet.

This data is freely available via the Internet on a global scale, with a resolution of 0.25°, i.e. around 25km at the equator, and a minimum time step of one hour.

Visual3 : Open-source GIS

Last but not least, a wealth of free cartographic data is now available from GIS portals, whether global, national or regional, and many open-source tools are available to exploit them.

These data form the basic foundation needed to analyze climate change and its impact on WASH infrastructures.

In four steps, I'll show you how to move from open climate data and GIS to the analysis of present and future climate risks that are essential for WASH projects.

# Analyze the context and identify relevant climate hazards

Visual4 : Climate context analysis - CCKP

The first step is to analyze the climatic context of the project area.

Numerous online data sources can be used, including the World Bank's Climate Change Knowledge Portal and GFDRR's ThinkHazard site, which provide a quick screening of climatic hazards and therefore of the climatic variables to be considered.

For water and wastewater, the recurring hazards are generally rain and river flooding, marine submersion, drought (and seasonal depletion), heat-humidity waves (staff safety, service continuity) and landslides.

Visual5 : Analysis framework - IPCC

It is also essential to establish an analysis framework, in line with that set out by the IPCC in its 6th report, in order to analyze future climate projections.

Analysis of future trends requires the definition of one or more analysis horizons, depending on the life span of the project in question.

Visual6 : SSPs

It is also necessary to define one or more climate scenarios to be analyzed. Consideration of the SSP5-8.5 scenario is strongly recommended, as observed climate trends since 2014, the reference date for the projections in the 6thIPCC report, are close to those projected under this scenario.

This initial analysis should enable you to identify the hazards that threaten your drinking water supply and wastewater treatment projects.

# Knowing which data to use

## Climatic data

Visual7 : Climatic data

Climate data is the scientific foundation of any analysis of the impact of climate change. Their quality, spatial and temporal resolution and traceability are essential to ensure robust results.

In Kenya, several levels of data can be combined:

Observed local data: Kenya Meteorological Department (KMD) weather stations provide time series on rainfall, temperature and sometimes evapotranspiration. Although often heterogeneous and incomplete, they are an indispensable reference for calibrating regional models and validating observed trends.

Simulated regional data (RCM): Models such as RegCM4 (CORDEX-Africa) or CRCM5 enable global projections to be brought down to a scale of 10 to 25 km, better suited to engineering needs. These models incorporate the complexity of monsoon regimes, interannual variability and the marked orographic effects of the Rift Valley.

Global data (reanalysis and projections): The ERA5 database, available for example from the Copernicus Climate Data Store, and the CMIP6 database, from the 6thIPCC report, provide continuous information from 1940 to 2100.

The variables extracted (rainfall, temperature, humidity, wind, radiation, etc.) are essential for calculating the climatic indices on which the analysis is based.

Once harmonized and processed (using Python scripts and open-source GIS tools such as QGIS, for example), these data can be used to produce temporal rasters at a resolution of 25km, containing past and projected hourly or daily data for the climate scenario(s) under consideration.

These data are essential for analyzing the impact of past and future climate on WASH infrastructures, and we'll come back to them when we examine climate indicators.

## Territorial map data

Visual8 : Cartographic data

Cartographic data can be used to spatialize risks and assess the vulnerability of infrastructures and populations.

They fall into several complementary categories:

Physical data relating to topography, hydrography, geology, land use, vegetation cover, etc. This information can be obtained via various portals and is used to model runoff, accumulation zones and erosion risks.

Socio-economic and institutional data, notably relating to population, poverty, access to services or water governance. Sources such as WorldPop, UN OCHA or national portals (Kenya Open Data) can be used to assess social vulnerability and adaptive capacity.

WASH infrastructure data: existing networks, water points, treatment plants, hydraulic structures. This data often comes from territorial diagnostics, GPS surveys or national agency databases such as WASREB and Water Services Boards.

The aim is generally to collect data from the local level, i.e. data available from local and national stakeholders, and then to fill in the gaps using free data available online. Numerous sites, such as Africa GeoPortal and Digital Earth Africa, provide access to such data.

These data are essential for analyzing the vulnerability of the territory and WASH infrastructures to climatic hazards.

# Choosing the right indicators

Visual9 : CC indicators

In line with the framework defined by the IPCC, we now need to define the relevant indicators that will enable us to qualify the exposure and vulnerability of planned developments to climate hazards, in order to deduce a level of impact in the present and future for the projected climate horizons and scenarios.

## To qualify exposure to climatic hazards

Visual10 : Climate hazards

With regard to exposure, the aim is to move from a mass of raw daily or hourly climate data to a small number of readable indicators, calculated over 10 or 20-year periods, to describe past trends in extremes and seasonality, characterize plausible futures by scenario, and finally link these signals to hazards impacting WASH infrastructures and to recommendations in terms of design and maintenance.

These indicators must be simple, traceable and useful. They are generally defined in such a way as to give a score from 1 to 4, the scale being defined at this stage, as is the analysis period. It's important to remember that a good indicator is one that guides a rule or a choice of design and adapted maintenance.

Indicators are generally calculated on the basis of parameters derived from past and future climatic data, such as the maximum daily total to analyze hydraulic stress, turbidity and network overflows. The maximum number of consecutive dry days can also be used to characterize pressure on resources or storage, as well as dry-season service. The so-called SPEI moisture index is also very useful for analyzing water deficit or surplus for planning purposes in water resource and demand management.

These indicators are estimated for current and future conditions for the climate horizons and scenarios considered.

## To qualify vulnerability to climatic hazards

Visual11 : WASH equipment

In parallel, it is necessary to qualify vulnerability to climatic hazards, which depends on many factors.

In the context of a WASH project, the level of vulnerability will depend on the type of development planned and the hazard concerned, as well as other factors such as soil texture or topographical features.

The aim here is to define a level of vulnerability, generally on a scale of 1 to 4, for each type of development and each category of hazard studied, based on the territorial mapping data collected.

## To qualify the level of climatic risk

Visual12 : Exposure - vulnerability cross-reference

Climate risk levels must then be estimated following the approach recommended by the IPCC, by cross-referencing exposure and vulnerability to climate hazards to estimate impact.

This cross-referencing is based on a cross-referencing grid which, for each climatic hazard, defines the level of impact according to the degree of exposure and vulnerability of each development under consideration.

These cross-referencing rules must be simple, legible and justified.

In order to take into account the impact of climate change, the calculation is carried out for the current state and the future state for the climate horizons and scenarios considered.

# Conducting the analysis

## Analyze risks and the impact of climate change

Visual13 : Data analysis

Once the climatic and territorial data have been collected and the indicators defined, the analysis phase consists in producing spatially explicit results that can be used directly by engineers, planners and decision-makers.

Based on historical and projected data, climate indicators are calculated for each analysis period (past, 2050 horizon, 2100 horizon).

Processing is generally carried out using Python scripts, which calculate the indices and then produce normalized rasters at a scale of 25 km or less.

Visual13 : Hazard map

These indicators are then translated into hazard maps by intensity class. The resulting maps can be used, for example, to visualize the territory's exposure to floods, droughts and heat waves according to the different climate scenarios considered.

Visual14 : Climate risk map

The next step is to cross-reference the hazard maps with the location of WASH facilities to estimate their exposure and then consider the vulnerability of each facility to calculate the level of impact for each hazard, through a risk rating.

This cross-referencing is generally carried out automatically in a GIS using scripts, to obtain a map of exposure to climatic risks.

## Making informed recommendations

These results can then be used to prioritize adaptation needs and guide investment priorities for WASH projects.

Recommendations can cover the technical design of facilities, operational management and the sustainability of services.

For example, for water supply, this may involve adjusting the sizing of boreholes and water towers to anticipate rainfall variability, reinforcing the protection of catchments against flooding or silting, or integrating more flexible storage systems.

In the wastewater sector, recommendations are often aimed at improving storm drainage, adapting networks to more intense rainfall events, or promoting decentralized solutions that are more resilient to flooding.

In terms of management, the emphasis is on preventive maintenance, redundancy of essential services and local capacity building to ensure infrastructure sustainability.

Finally, water governance must be integrated into the approach to encourage community participation, strengthen coordination between stakeholders and anticipate future water and sanitation needs in a changing climate.

By combining these technical, organizational and institutional levers, WASH projects become not only more robust, but also more equitable and sustainable in the face of future climate challenges.

# Conclusion & key messages

Climate analysis applied to WASH projects in Kenya demonstrates that data is the new foundation of resilience. Advances in climate science, the widespread use of open data and the rise of digital tools now offer the means to integrate climate at the heart of infrastructure planning.

Three key lessons emerge:

Open, interoperable data is a major opportunity for development players. By combining ERA5, CMIP6, CORDEX and African GIS portals, it is now possible to conduct rigorous analyses even in contexts with low local data density.

Early integration of climate risk into the project cycle is a lever for sustainability. Designing a water treatment plant, borehole or network with future projections in mind means investing in service continuity and user safety.

Data governance and transparency of results are essential for concerted action. Disseminating vulnerability maps and recommendations on collaborative platforms promotes coordination between engineers, local authorities and landlords, and reinforces collective responsibility in the face of climate change.

In short, developing a WASH climate analysis means moving from reaction to prevention. It also means building the engineering of the future: engineering based on data, on science, and on an ethical vision of sustainable development - one that aims to guarantee every community access to water and sanitation that can withstand the shocks of a changing climate.